ATTACHMENT I

Chronic Reference Exposure Levels (RELs) 
Finalized by OEHHA January 2001

Chemical Name
CAS No.
Chronic REL, µg/m3 *

Acrolein
107-02-8
6.00E-2

Arsenic 
& arsenic compounds
7440-38-2
3.00E-2

Butadiene
106-99-0
2.00E+1

Cadmium
& cadmium compounds
7440-43-9
2.0E-2

Carbon tetrachloride
56-23-5
4.00E+1

Chlorine dioxide
10049-04-4
6.00E-1

Chlorobenzene
108-99-7
1.00E+3

Chromium, hexavalent
soluble compounds
except chromic trioxide

2.00E-1

Chromic trioxide (as chromic acid mist)

2.00E-3

Cresol mixtures
1319-77-3
6.00E+2

Dichlorobenzene (1,4-)
106-46-7
8.00E+2

Dichloroethylene (1,1-)
73-35-4
7.00E+1

Dimethylformamide (N,N-)
68-12-2
8.00E+1

Epichlorohydrin
106-89-8
3.00E+0

Epoxybutane (1,2-)
106-88-7
2.00E+1

Ethylene dichloride
107-06-2
4.00E+2

Ethylene oxide
75-21-8
3.00E+1

Glutaraldehyde
111-30-8
8.00E-2

Hydrazine
302-01-2
2.00E-1

Methylene diphenyl isocyanate
101-68-8
7.00E-1

Phthalic anhydride
85-44-9
2.00E+1

Toluene diisocyanate (2,4-)
584-84-9
7.00E-2

Toluene diisocyanate (2,6-)
91-08-7
7.00E-2

*
Lower values are more stringent.

